# Persona Communication Matrix

This page maps communication between SMD personas and the core destinations they reach, together with the controls applied to each permitted flow. The principle is unchanged from the rest of this strategy: inspect once at the optimal point, never at the same depth twice.

## At-a-Glance Matrix

Rows are the source (the side that opens the connection); columns are the destination. The tag in each permitted cell shows where the flow is deep-inspected.

| ↓ source · → dest | Warehouse ops | Automation | Site systems | WMS — cloud | WMS — DC | Internet / SaaS |
| --- | --- | --- | --- | --- | --- | --- |
| Warehouse ops | — | ✓ Site | ✓ Site | ✓ Hub | ✓ Hub | ✓ SSE |
| Automation | △ Site | — | ✓ Site | ✓ Hub | ✓ Hub | ✕ |
| Site systems | ✓ Site | ✓ Site | — | ✓ Hub | ✓ Hub | △ SSE |
| WMS — cloud | ✓ Hub | ✓ Hub | ✓ Hub | — | △ Hub | ✓ Cloud |
| WMS — DC | ✓ Hub | ✓ Hub | ✓ Hub | △ Hub | — | ✓ DC |
| Internet / SaaS | ✕ | ✕ | ✕ | △ Cloud | △ DC | — |

| Allowed | Exception only | Denied (default) |
| --- | --- | --- |

Tag key — site: stateful control at the site (Tier 1). hub: deep inspection on the WAN leg at the hub (Tier 2), with micro-segmentation at the workload (Tier 3). SSE: user internet via the cloud proxy (Tier 0). cloud / DC: server-initiated egress handled closest to the workload, not back through the user proxy.

3rd-party and guest zones are excluded from this view: both are denied by default and configured bespoke per site, and guest zones are internet-only and fully isolated. Two cells are provisional pending confirmation — automation-to-internet is shown as denied (any vendor telemetry would be a documented exception), and WMS cloud-to-DC as an exception (treated as migration data-sync).

## Detailed Enforcement Matrix

The same flows expressed in full — the enforcement path each one traverses and the controls applied along it.

| Source | Destination | Communicate? | Enforcement path | Controls applied |
| --- | --- | --- | --- | --- |
| Automation | SMD Site Systems | Yes, approved local service flows only | Automation zone → Tier 1 site enforcement → SMD site systems | Tier 1 zone separation, ACL allow-listing, stateful inspection, permit/deny logging. No default site DPI/IPS. |
| Automation | WMS Cloud | Yes, approved application flows only | Automation zone → Tier 1 → Tier 2 hub → Tier 3 cloud workload | Tier 1 ACL/SPI, Tier 2 DPI/IPS/anti-malware, Tier 3 micro-segmentation, centralized logging. |
| Automation | WMS DC | Yes, transitional approved flows only | Automation zone → Tier 1 → Tier 2 hub / SMD DC | Tier 1 ACL/SPI, Tier 2 or SMD DC deep inspection, IPS/IDS, anti-malware, centralized logging. Retire as WMS DC dependency is removed. |
| SMD Warehouse Ops | Automation | Yes, approved operational workflows only | Warehouse Ops zone → Tier 1 site enforcement → Automation zone | Tier 1 zone separation, source/destination/port allow-listing, stateful inspection, permit/deny logging. No broad lateral access. |
| SMD Warehouse Ops | WMS Cloud | Yes, approved WMS access only | Warehouse Ops zone → Tier 1 → Tier 2 hub → Tier 3 cloud workload | Tier 1 ACL/SPI, Tier 2 DPI/IPS/anti-malware, Tier 3 workload micro-segmentation, centralized logging. |
| SMD Warehouse Ops | WMS DC | Yes, transitional approved WMS access only | Warehouse Ops zone → Tier 1 → Tier 2 hub / SMD DC | Tier 1 ACL/SPI, Tier 2 or SMD DC inspection, IPS/IDS, anti-malware, centralized logging. Retire as workloads migrate to cloud. |
| Any site source | Other SMD site networks | Denied by default | Site-to-site traffic blocked unless approved exception exists | RFC1918 deny posture, exception-based ACLs, hub inspection for approved exceptions, time-bound approval, audit logging. |
| Warehouse Ops user traffic | Internet / SaaS | Yes, via approved cloud proxy/SSE path | Tier 1 permits tunnel/direct path to Tier 0 cloud proxy/SSE; hub bypassed | Tier 0 SWG, URL filtering, SSL/TLS inspection where enabled, malware scanning, DLP, DNS security, user/SaaS logging. |